Moon Satellite

for Only Watch 2019

Geneva, September 23rd 2019

Moon Satellite

For Only Watch 2019

Technical specifications

Name: 	Moon Satellite for Only Watch 2019

Reference:	OW19

Functions:	Wandering hour satellite, Minutes, Spherical moon phase indication at 6 o’clock, Power reserve situated on the back

Movement:	DB2105

Type:	Mechanical hand-wound movement

Adjustment:	Winding, setting the time and spherical moon phase adjustment by means of the crown (3 positions)

Technical features of calibre DBUR2105

Number of parts:	336

Jewelling:	37 jewels

Diameter:	30 mm

Power reserve: 	4 days, ensured by a self-regulating twin barrel

De Bethune Innovation (2004)

Specificities:

Hour satellite indication designed by URWERK with triple base plate in ARCAAP; satellite in aluminum mounted on Geneva crosses in beryllium bronze. Hand painted SuperLuminova index.

Titanium balance wheel with white gold inserts, optimised for temperature differences and air penetration

De Bethune Patent (2016)

"De Bethune" balance-spring with flat terminal curve

De Bethune Patent (2006)

Silicon escape wheel

Spherical moon phase indication accurate to a degree of one lunar day every 122 years – De Bethune Patent (2004)

Triple pare-chute shock-absorbing system

De Bethune Innovation (2005)

Frequency: 	28,800 vibrations per hour

Adornment: 	Concave plate covers and mirror-polished barrels bridge

Open worked and hand sailed barrels

Steel parts beveled and polished by hand

Case and strap

Case material:	Polished grade 5 titanium

Case width:	43 mm

Case thickness:	13.3 mm

Lugs: 	Short or long floating lugs in polished grade 5 titanium

De Bethune Patent (2006)

Crystal: 	In sapphire crystal (1800 Vickers hardness) with double anti-reflective coating

Case back: 	Screwed case back in grade 5 polished titanium

Sapphire crystal (1800 Vickers hardness) with double anti-reflective coating

Water resistance: 	3 ATM

Strap: 	Extra-supple alligator leather, alligator lining

Buckle:	Pin buckle in polished grade 5 titanium